THE STRATEGIC POTENTIAL OF GHANA'S GAS-TO-POWER SECTOR FOR ENERGY SECURITY AND ECONOMIC GROWTH

EXECUTIVE SUMMARY

Ghana's gas-to-power sector stands at a critical inflection point. Natural gas now dominates the country's electricity generation mix, accounting for approximately 65.8% of total supply in 2025, with gas-fired thermal plants providing 3,676 MW of installed capacity (67% of total capacity) against a system peak demand of 4,125 MW. The sector's strategic importance is underscored by Ghana's proven natural gas reserves of approximately 48 billion cubic meters (bcm), production of 3.5 bcm in 2024, and a gas-to-power value chain anchored by three producing offshore fields (Jubilee, TEN, Sankofa), the Atuabo Gas Processing Plant, the ENI Offshore Receiving Facility, and the operational Tema LNG Terminal (1.7 mtpa).

KEY OPPORTUNITIES:

- Domestic gas production tripling since 2018, reducing import dependence

- Tema LNG Terminal providing supply diversification and price hedging

- Strong industrial demand growth (non-power gas demand projected at 13,788 MMscf in 2025)

- Regional electricity export potential via WAPP interconnections

- $5 billion Qatar fertilizer plant deal at Atuabo Petroleum Hub

FIVE PRIORITY RECOMMENDATIONS:

- Implement comprehensive IPP contract renegotiation to eliminate take-or-pay obligations and align capacity payments with actual dispatch.

- Establish a Gas Supply Security Reserve using Tema LNG storage to buffer against domestic supply disruptions.

- Accelerate domestic gas field development (Pecan, Eban-Akoma) and expand pipeline interconnectivity between western and eastern corridors.

- Launch competitive gas-to-industry auction mechanism to monetize stranded gas and attract petrochemical investment.

- Integrate gas-fired generation with utility-scale renewables and battery storage to create a flexible, low-carbon power system aligned with 2060 net-zero targets.

  - STRATEGIC OVERVIEW OF GHANA'S GAS-TO-POWER SECTOR

1.1 Resource Base and Production

Ghana's natural gas sector has evolved rapidly since first commercial production in 2010. The country's proven gas reserves are estimated at approximately 48 bcm, with the majority concentrated in three offshore fields:

- Jubilee Field: Discovered in 2007, operated by Tullow Oil (35.48%), with Kosmos Energy (24.08%), PetroSA (4.85%), GNPC (13.64%), and Jubilee Oil Holdings (22.9%). Associated gas production averaged 233.29 MMscf/d in 2024, with crude oil output of 31.85 million barrels (87,258 bbl/d).

- Ten Field (Tweneboa-Enyenra-Ntomme): Operated by Tullow Oil (54.8%), with Kosmos Energy (17%), PetroCI (11%), GNPC (15%), and Anadarko (2.2%). Associated gas production averaged 159.54 MMscf/d in 2024, with crude output of 6.78 million barrels (18,588 bbl/d).

- Sankofa Field (OCTP/Sankofa-Gye Nyame): Operated by ENI (44.44%), with Vitol (33.33%) and GNPC (22.22%). Non-associated gas production averaged 237.40 MMscf/d in 2024, with crude output of 9.61 million barrels (26,319 bbl/d). Sankofa is Ghana's primary non-associated gas field and the largest contributor to domestic lean gas supply.

Total raw gas production from the three fields reached 280,441 MMscf in 2024, with 115,455 MMscf exported to processing facilities. Gas production has tripled since 2018, fundamentally transforming Ghana's energy landscape.

1.2 Gas-to-Power Value Chain

Ghana's gas-to-power value chain comprises:

UPSTREAM: Three producing offshore fields (Jubilee, TEN, Sankofa) with associated and non-associated gas. The Pecan field and Eban-Akoma discoveries represent future supply growth potential.

MIDSTREAM:

- Atuabo Gas Processing Plant (AGPP): Processes raw associated gas from Jubilee and TEN fields under a substitution agreement. Supplied 16,954 MMscf of lean gas in H1 2024 (avg. 85.79 MMscfd).

- ENI Offshore Receiving Facility (ORF): Receives non-associated gas from Sankofa field. Supplied 42,291 MMscf in H1 2024 (avg. 233.98 MMscfd).

- Tema LNG Terminal: Operational since 2023, 1.7 mtpa regasification capacity, $350 million investment. Provides supply diversification and price hedging.

- West African Gas Pipeline (WAGP): Imports from Nigeria, averaging 62.86 MMscfd in H1 2024 (exceeding 60 MMscfd projection).

DOWNSTREAM: Thermal power plants totaling 3,676 MW installed capacity, representing 69.9% of Ghana's grid-connected generation. Major plants include Sunon Asogli (560 MW), Karpower (470 MW), AKSA (370 MW), Cenpower (360 MW), TAPCO/TICO (670 MW combined), and others.

1.3 Strategic Importance

Gas-to-power is strategically vital for Ghana for five reasons:

- Energy Security: Gas-fired generation provides baseload and mid-merit capacity that is less vulnerable to hydrological variability than the 1,584 MW hydro fleet (Akosombo, Bui, Kpong), which faces recurrent drought risks.

- Fuel Diversification: The shift from light crude oil and heavy fuel oil to natural gas has reduced fuel costs and emissions. In 2025, 97.5% of thermal fuel expenditure ($1,217.54 million of $1,248.23 million) is allocated to natural gas procurement.

- Industrialization: Reliable gas supply enables energy-intensive manufacturing, mining, and emerging petrochemical industries. The $5 billion Qatar fertilizer plant at Atuabo exemplifies this potential.

- Regional Integration: Ghana's grid interconnections with Côte d'Ivoire, Togo, and Burkina Faso position it as a potential regional electricity hub, with surplus export capacity during favorable supply periods.

- Transition Bridge: Natural gas offers a lower-carbon alternative to coal and oil for baseload generation while renewable capacity scales up (currently only 132 MW, or 2.4% of installed capacity).

Table 1: Ghana's Gas-To-Power Profile

| Indicator | Latest Value | Strategic Significance |
| --- | --- | --- |
| Proven Gas Reserves | ~48 bcm | Sufficient for 15–20 years at current production rates |
| 2024 Gas Production | 3.5 bcm | Tripled since 2018; 66% from Sankofa field |
| 2025 Gas Demand (Power) | 151.4 TBtu | 133,977 MMscf; dominates thermal generation fuel mix |
| Installed Generation Capacity | 5,507 MW | 45% increase since 2016; thermal share at 67% |
| Thermal (Gas) Capacity | 3,676 MW | 69.9% of grid-connected capacity; backbone of reliability |
| Hydro Capacity | 1,584 MW | 29% of capacity; vulnerable to rainfall variability |
| Renewable Capacity | 132 MW | 2.4% of capacity; far below 1.5 GW 2030 target |
| System Peak Demand (2025) | 4,125 MW | 4.4% annual growth; industrial demand driving expansion |
| Reserve Margin (2025) | 18% (nominal) | Drops to ~12% during maintenance/fuel contingencies |
| WACOG (2025) | $8.04/MMBtu | High by regional standards; impacts tariff affordability |
| Tema LNG Capacity | 1.7 mtpa | Provides supply diversification and price hedging |
| WAGP Import (Nigeria) | 73–120 MMscfd | Contractual minimum; reliability tied to payment guarantees |
| Electricity Access Rate | 89.4% (2024) | Near-universal urban (93.8%); rural gap remains (70%) |
| Sector Debt | ~$5.6 billion | Critical constraint on investment and supply security |
| GDP Growth (2024) | 5.7% | Energy-intensive industry drove acceleration from 3.1% |
| Industry Share of GDP (2024) | 31.52% | Above global average (26.19%); gas demand correlates |

Ghana's gas-to-power sector has achieved impressive production growth but faces a structural mismatch between supply reliability and demand growth. The 2025 gas supply shortfall projections reaching 102 MMscfd during November maintenance—demonstrate that production growth has not translated into supply security. The sector's strategic value is contingent on resolving infrastructure bottlenecks and financial viability.

2. CONTRIBUTION OF GAS-TO-POWER TO ENERGY SECURITY

2.1 Electricity Reliability and Generation Capacity

Natural gas has become the cornerstone of Ghana's electricity reliability. In 2025, thermal plants (primarily gas-fired) are projected to generate 16,997 GWh (65.8% of total supply), compared to 8,561 GWh (33.1%) from hydro and 196 GWh (0.8%) from renewables. This represents a fundamental shift from the hydro-dependent system of the 1990s.

The gas-fired fleet provides critical grid services:

- Baseload Capacity: Sunon Asogli (560 MW), Cenpower (360 MW), and TAPCO/TICO (670 MW combined) provide continuous baseload generation.

- Mid-Merit Flexibility: Karpower (470 MW, barge-mounted), AKSA (370 MW), and Kpone (220 MW) offer dispatch flexibility to match demand variations.

- Spinning Reserve: Dual-fuel capability (80% of thermal plants can switch to LCO/DFO) provides strategic reserve during gas supply interruptions.

2.2 Fuel Diversification and Import Dependence

Ghana's gas supply portfolio has three pillars:

Domestic Supply (Projected 2025: 116,840 MMscf):

- Jubilee/TEN via AGPP: ~32,589 MMscf (100 MMscfd nominal)

- Sankofa via ENI ORF: ~84,251 MMscf (246 MMscfd nominal)

Imported Supply:

- WAGP (Nigeria): Minimum 73 Mmscfd, Up To 120 Mmscfd Contractually

- Tema LNG: Spot and medium-term contracts, 1.7 mtpa capacity

This three-source portfolio represents significant diversification from the pre-2018 era when Ghana relied heavily on imported liquid fuels and Nigerian gas. However, import dependence remains material. In H1 2024, WAGP supplied 11,377 MMscf (16% of total lean gas supply), and the Tema LNG Terminal provides critical backup during domestic shortfalls.

2.3 Reduction in Power Shortages

Ghana has made substantial progress in reducing power shortages since the 2012–2016 "dumsor" crisis. The 2025 Energy Outlook projects reserve margins between 12% and 28%, with system peak demand of 4,125 MW against dependable capacity of 4,856 MW. However, available capacity drops to 4,656 MW when factoring in planned maintenance and fuel supply contingencies, creating tight margins during peak periods.

The stabilization of supply has enabled Ghana to resume electricity exports to neighboring countries, with 8.5% of generation allocated to export in 2024. However, Energy Minister John Abdulai Jinapor has cautioned that structural debt remains a critical vulnerability.

2.4 Current Limitations

Despite progress, significant energy security limitations persist:

Supply Volatility: Planned maintenance on Jubilee FPSO (15 days in March 2025), Sankofa shutdowns (8 days in September, 15 days in November), and WAGP maintenance create cumulative deficits of up to 102 MMscfd. The 2025 outlook indicates supply will be insufficient to meet demand from January through August.

Infrastructure Concentration: 52.5% of dependable thermal capacity is concentrated in the Tema enclave (Southeast), creating geographic vulnerability. The recent relocation of Ameri plant units to the Ashanti enclave (Anwomaso, 250 MW) improves resilience but remains insufficient.

Financial Constraints: ECG's $4.2 billion arrears to power producers and gas suppliers have triggered supply disruptions. Sunon Asogli shut down temporarily in October 2024 over $259 million in unpaid arrears.

Hydro Vulnerability: The 1,584 MW hydro fleet (Akosombo 1,020 MW, Bui 404 MW, Kpong 160 MW) remains vulnerable to rainfall variability. In 2024, low water levels constrained Akosombo output, increasing gas-fired dispatch.

TABLE 2: Energy Security Assessment

| Indicator | Current Status | Implication |
| --- | --- | --- |
| Generation Adequacy | 5,507 MW / 4,125 MW | 18% nominal reserve; tight during contingencies |
| Fuel Diversity | 3-source gas portfolio | Reduces single-point failure risk |
| Supply Reliability | Frequent maintenance | 102 MMscfd max deficit in 2025 |
| Grid Stability | Single-axis dominance | Tema enclave concentration risk |
| Import Dependence | 16% via WAGP + LNG | Manageable but price-exposed |
| Financial Sustainability | $5.6B sector debt | Payment arrears triggering supply disruptions |
| Backup Fuel Capability | 80% dual-fuel plants | LCO/DFO available at higher cost |
| Transmission Redundancy | Limited inter-enclave | Ashanti link improves but gaps remain |
| Storage Capacity | Minimal (LNG tankage) | No strategic gas storage; LNG provides buffer |
| Demand-Side Management | Underdeveloped | Peak demand growing 4.4% annually |

Ghana's energy security has improved operationally but remains financially fragile. The combination of supply volatility, geographic concentration, and unsustainable sector debt creates a compound risk. A single major supply disruption during a period of fiscal constraint could trigger cascading failures across the value chain. Policymakers should prioritize financial restructuring alongside infrastructure hardening.

3. ECONOMIC GROWTH AND INDUSTRIAL DEVELOPMENT POTENTIAL

3.1 Sectoral Economic Impact

Ghana's GDP growth accelerated to 5.7% in 2024 (from 3.1% in 2023), with industry (including mining, quarrying, and construction) contributing significantly. The industrial sector's share of GDP reached 31.52% in 2024, above the global average of 26.19%. Reliable gas-fired electricity is a critical enabler of this growth.

Manufacturing: Ghana's manufacturing sector consumes approximately 18.6% of total final energy. Key sub-sectors (textiles, food processing, cement, steel) require continuous power supply. Gas-to-power reduces manufacturing energy costs compared to liquid fuel alternatives, improving competitiveness. The $5 billion Qatar-Al Jedad fertilizer plant at Atuabo (2,000 jobs, commissioning 2028) will be a major gas offtaker and industrial anchor.

Mining: Ghana is Africa's largest gold producer and a significant bauxite and manganese exporter. Mining operations (5.7% of electricity consumption) require reliable power for extraction, processing, and refining. Grid-connected mines benefit from gas-fired baseload, while off-grid operations currently rely on diesel. The Genser embedded generation units (194 MW across five mine sites) demonstrate the mining sector's gas demand potential.

Agriculture: While direct agricultural gas use is limited, the fertilizer plant project will transform Ghana from a net importer (554,239 tons in 2024, third-largest in West Africa) to a regional producer. Natural gas is the primary feedstock for urea and ammonia production.

Petrochemicals: Ghana's Gas Master Plan identifies petrochemicals (methanol, dimethyl ether) as long-term utilization options. However, viability requires world-scale plants (5,000 tpd methanol) and gas prices below $4.8/MMBtu—well below current WACOG levels. The fertilizer plant represents a more near-term petrochemical opportunity.

Small And Medium Enterprises (SMES): SMEs constitute over 90% of Ghanaian businesses but face high electricity costs and unreliable supply. Gas-to-power expansion, if translated into affordable tariffs, could unlock SME productivity and employment creation.

3.2 Economic Multiplier Effects

Reliable electricity supply generates substantial economic multipliers:

Direct Effects: Power sector investment creates construction, operations, and maintenance employment. The Tema LNG Terminal ($350 million) and planned extensions employ hundreds directly.

Indirect Effects: Manufacturing competitiveness improvements attract foreign direct investment (FDI). Ghana's FDI inflows are correlated with power sector performance. The mining sector's expansion (new gold concessions, lithium exploration) depends on grid reliability.

Induced Effects: Household productivity gains from reduced outages enable small business operation, extended trading hours, and improved education outcomes (electricity access now at 89.4% nationally).

3.3 Foreign Direct Investment and Employment

The gas-to-power sector has attracted significant FDI:

- Sunon Asogli: Chinese investment (Shenzhen Energy 60%, CAD Fund 40%), 560 MW operational, 508 MW CCGT expansion planned (total >1 GW).

- Karpower: Turkish investment, 470 MW barge-mounted plant.

- Cenpower: 360 MW combined cycle plant.

- Tema Lng: Helios Investment Partners and GNPC joint venture.

Employment creation spans upstream (oilfield services, offshore operations), midstream (pipeline construction, plant operations), and downstream (power plant operations, industrial utilization). The Qatar fertilizer project alone projects 2,000 direct jobs.

TABLE 3: ECONOMIC IMPACT ASSESSMENT

| Sector | Current Contribution | Potential Impact of Expanded Gas-to-Power |
| --- | --- | --- |
| Manufacturing | 31.52% of GDP | Reduced energy costs; improved competitiveness; FDI attraction |
| Mining | 5.7% of electricity | Expanded production; new concessions; lithium processing |
| Agriculture | 554,239t fertilizer imports | Domestic fertilizer production; food security; export earnings |
| Petrochemicals | Minimal | Fertilizer anchor; methanol/DME long-term potential |
| SMEs | 90%+ of businesses | Productivity gains; employment; reduced coping costs |
| Power Export | 8.5% of generation | Regional hub status; forex earnings; WAPP integration |
| Employment | Thousands direct | Tens of thousands indirect via industrialization |
| FDI | Multi-billion $ | Continued attraction contingent on sector financial health |

The economic multiplier potential of Ghana's gas-to-power sector is substantial but unrealized. The binding constraint is not gas availability but rather the sector's financial unsustainability, which prevents tariff levels that would support industrial competitiveness while covering costs. The Qatar fertilizer deal demonstrates that anchor industrial projects can proceed despite sector challenges, but broad-based industrialization requires systemic financial reform.

4. INFRASTRUCTURE AND INVESTMENT ASSESSMENT

4.1 Gas Processing and Pipeline Infrastructure

ATUABO GAS PROCESSING PLANT (AGPP):

- Status: Operational since 2015

- Function: Processes associated gas from Jubilee/TEN fields

- 2024 Performance: Supplied 16,954 MMscf in H1 (avg. 85.79 MMscfd, below 97 MMscfd projection)

- Constraint: Limited by raw gas availability from Jubilee/TEN; processing capacity underutilized during field maintenance

- Investment Priority: MEDIUM (expand processing capacity contingent on upstream production growth)

ENI OFFSHORE RECEIVING FACILITY (ORF):

- Status: Operational

- Function: Receives non-associated gas from Sankofa field

- 2024 Performance: Supplied 42,291 MMscf in H1 (avg. 233.98 MMscfd, near 237.5 MMscfd projection)

- Constraint: Dependent on Sankofa field reliability; field declining (-13% projected for 2025)

- Investment Priority: HIGH (field redevelopment and compression needed)

WEST AFRICAN GAS PIPELINE (WAGP):

- Status: Operational since 2011

- Function: Imports gas from Nigeria (Escravos-Lagos Pipeline System)

- 2024 Performance: 62.86 MMscfd average (H1), exceeding 60 MMscfd projection

- Constraint: Supply reliability tied to Nigerian gas availability and Ghana's payment guarantees; contractual minimum 73 MMscfd

- Investment Priority: HIGH (expand contractual volumes; secure payment mechanism)

TEMA LNG TERMINAL:

- Status: Operational since 2023

- Capacity: 1.7 mtpa regasification

- Cost: $350 million

- Function: Supply diversification, price hedging, strategic reserve

- Constraint: Marine access reliability; downstream pipeline capacity; commercial offtake arrangements

- Investment Priority: HIGH (optimize utilization; expand storage)

4.2 Power Generation Assets

Ghana's thermal fleet comprises 3,676 MW of installed capacity across multiple technologies and ownership structures:

COMBINED CYCLE GAS TURBINE (CCGT):

- Sunon Asogli: 560 MW (operational), 508 MW expansion planned

- Cenpower: 360 MW

- TAPCO/TICO: 670 MW combined (dual-fuel)

SIMPLE CYCLE GAS TURBINE (SCGT) / OPEN CYCLE:

- Kpone: 220 MW

- Anwomaso: 250 MW (recently relocated Ameri units)

- Early Power: 200 MW

- Various others

DUAL-FUEL / BARGE:

- Karpower: 470 MW (barge-mounted, gas/HFO)

- AKSA: 370 MW (HFO primary, gas-capable)

EMBEDDED GENERATION:

- Genser: 194 MW across five mine sites (grid + embedded)

4.3 Financing Requirements and Private Sector Participation

The Ghana Energy Sector Recovery Program (ESRP), supported by the World Bank, estimates the sector requires approximately $1.085 billion in investment to restore financial viability. Key financing needs include:

- GRIDCo system strengthening: Technical loss reduction, dispatch efficiency

- ECG distribution strengthening: Commercial loss reduction, smart metering

- IPP contract restructuring: Elimination of take-or-pay obligations

- Gas infrastructure expansion: Pipeline interconnectivity, storage

Private sector participation is substantial but strained:

- IPPs provide 65% of dependable capacity

- Take-or-pay contracts have accumulated unsustainable arrears

- PPA prices for gas-fired plants average $12.35/kWh—significantly above Nigeria's $7.3/kWh average

The World Bank's ESRP projects sector shortfalls could be reduced from $1.319 billion (2022) to $1.03 billion (2024) and $886 million (2027) if reform actions are implemented. However, without reform, cumulative deficits could exceed $9 billion by 2026.

TABLE 4: INFRASTRUCTURE ASSESSMENT

| Infrastructure | Current Status | Constraint |
| --- | --- | --- |
| Atuabo Gas Processing Plant | Operational | Underutilized; raw gas supply volatility |
| ENI ORF | Operational | Sankofa field decline (-13% in 2025) |
| Tema LNG Terminal | Operational (2023) | Marine access; downstream pipeline capacity |
| WAGP | Operational | Nigerian supply reliability; payment guarantees |
| East-West Pipeline | Partial | Limited interconnectivity between corridors |
| Gas Storage | None (strategic) | No buffer against supply disruptions |
| Tema Power Enclave | Concentrated | 52.5% of thermal capacity; vulnerability |
| Ashanti Enclave | Developing | Anwomaso 250 MW; needs expansion |
| GRIDCo Transmission | Operational | 3.9% losses; constraints on economic dispatch |
| ECG Distribution | Operational | 28.4% technical+commercial losses; $4.2B debt |

TABLE 5: INVESTMENT READINESS ASSESSMENT (1–5 Scale)

| Factor | Rating | Rationale |
| --- | --- | --- |
| Regulatory Certainty | 2 | Recent amendments (Energy Sector Levies Act 2025) improve revenue but ongoing IPP contract disputes and PURC tariff delays create uncertainty |
| Infrastructure Readiness | 3 | Core infrastructure exists but gaps in storage, interconnectivity, and distribution modernization limit expansion potential |
| Financing Availability | 2 | World Bank ESRP ($1.085B) committed but private capital constrained by sector debt and currency depreciation risks |
| Market Attractiveness | 3 | Strong demand growth (4.4% annually) and industrial potential attract investors, but tariff affordability caps returns |
| Institutional Capacity | 3 | GNPC, Ghana Gas, Energy Commission capable but under-resourced; coordination gaps between ministries |
| Political Stability | 4 | Stable democratic governance; 2024 election transition smooth; energy sector reform has cross-party support |

Ghana's gas-to-power infrastructure is operationally functional but financially distressed. The Tema LNG Terminal represents a strategic success, yet the absence of strategic gas storage and limited pipeline interconnectivity create vulnerability. Investment readiness is moderate at best; private capital will remain cautious until the ESRP demonstrates measurable financial improvement. The $5.6 billion sector debt is the single greatest impediment to infrastructure expansion.

5. STRATEGIC RISKS AND OPPORTUNITIES

5.1 Risk Assessment

Domestic Gas Supply Disruptions: Planned maintenance on Jubilee FPSO, Sankofa shutdowns, and WAGP maintenance will create cumulative deficits of up to 102 MMscfd in 2025. Unplanned disruptions (due to equipment failure, reservoir issues, or operational incidents) could exacerbate shortfalls. The Sankofa field's 14.6% decline in 2024 signals accelerating depletion risks.

Pipeline Constraints: The concentration of thermal capacity in the Tema enclave (52.5%) creates geographic vulnerability. Limited pipeline interconnectivity between western (Atuabo-Takoradi) and eastern (Tema) corridors constrains gas routing flexibility. Any disruption to the eastern corridor would disproportionately impact generation.

Financing Challenges: The $5.6 billion sector debt, ECG's $4.2 billion arrears, and unsustainable take-or-pay IPP contracts create a debt-investment spiral. Without ESRP implementation, cumulative deficits could exceed $9 billion by 2026, crowding out capital expenditure and triggering supply disruptions.

Regulatory Uncertainty: While the Energy Sector Levies (Amendment) Act 2025 and Environmental Protection Act 2025 provide improved frameworks, ongoing IPP contract disputes, PURC tariff delays, and the pending National Petroleum Authority Bill create uncertainty. The repeal of the Emissions Levy Act 2025 may signal reduced climate policy certainty.

Dependence On Imported Gas: WAGP imports (16% of supply) and LNG spot purchases expose Ghana to international price volatility and supply disruptions. Nigeria's gas availability is subject to domestic demand pressures and infrastructure constraints.

Climate Transition Risks: Ghana's Energy Transition and Investment Plan (2023) targets net-zero by 2060, requiring $550 billion in investment. International climate finance increasingly favors renewables over gas, potentially constraining gas-to-power financing. However, gas remains essential as a transition fuel given current renewable capacity of only 132 MW.

5.2 Opportunity Assessment

Domestic Gas Development: The Pecan field and Eban-Akoma discoveries offer significant reserve additions. ENI's agreement to increase supply by 30 MMscfd (effective July 2025) demonstrates operator commitment. However, development timelines (3–5 years) and capital requirements ($1–3 billion per field) present challenges.

Expansion Of Gas-Fired Generation: Sunon Asogli's planned 508 MW CCGT expansion would increase the company's capacity to over 1 GW. Additional CCGT capacity can be deployed relatively quickly (24–36 months) given existing gas infrastructure.

Industrialization: The Qatar fertilizer plant ($5 billion), petrochemical potential, and mining sector expansion create substantial non-power gas demand. Industrial gas demand is projected to reach 13,788 MMscf in 2025, up from 6,263 MMscf in H1 2024.

Regional Electricity Exports: Ghana currently exports 8.5% of generation. WAPP integration and interconnections with Côte d'Ivoire, Togo, Burkina Faso, and planned links to Mali and Niger create regional market opportunities. Ghana's relatively reliable supply positions it as a regional hub.

Private Investment: Chinese, Turkish, and private equity investment (Helios) has flowed into the sector. Further investment is contingent on financial restructuring and regulatory certainty. The Petroleum Hub development at Atuabo could anchor integrated investment.

Ecowas Energy Integration: MEDIUM): The West African Power Pool (WAPP) Master Plan envisions regional electricity trading. Ghana's gas-to-power capacity could support regional supply security, particularly as Nigeria's power sector struggles with gas availability.

Clean Energy Transition: Gas-fired generation provides flexible backup for renewable expansion. Ghana's target of 1.5 GW wind and solar by 2030 requires firm capacity. Combined cycle gas turbines with carbon capture readiness could align with long-term decarbonization.

6. BENCHMARKING GHANA AGAINST REGIONAL PEERS

Ghana's gas-to-power development can be assessed against four regional comparators:

NIGERIA:

- Gas Reserves: ~5.9 trillion cubic meters (Tcm) — world's 9th largest

- Gas-to-Power: Despite massive reserves, Nigeria generates only ~4,000 MW from gas due to infrastructure constraints, pipeline vandalism, and regulatory dysfunction. Power sector heavily reliant on diesel generators.

- Infrastructure: Escravos-Lagos pipeline feeds WAGP; domestic distribution constrained by aging infrastructure and security challenges.

- Regulatory Framework: Petroleum Industry Act 2021 reformed the sector but implementation remains incomplete.

- Investment Attractiveness: High resource potential undermined by governance and security risks.

- Energy Security: Chronic power shortages ("epileptic power") despite gas abundance.

CÔTE D'IVOIRE:

- Gas Reserves: ~1.0 Tcm (Baleine field major discovery)

- Gas-to-Power: ~70% of electricity from gas; Eni's Baleine Phase 2 (December 2024) added 70 MMscfd for power generation. Strong regional energy hub status.

- Infrastructure: Well-developed gas processing and power generation infrastructure.

- Regulatory Framework: Stable regulatory environment; strong government-IOC partnerships.

- Investment Attractiveness: High; Eni's net-zero upstream project demonstrates climate-aligned investment potential.

- Energy Security: Reliable supply; expanding export capacity to neighbors.

EGYPT:

- Gas Reserves: ~2.1 Tcm; Zohr field (30 Tcf) transformed supply outlook

- Gas-to-Power: ~70% of electricity from gas; 60+ GW installed capacity

- Infrastructure: Extensive pipeline network, LNG export terminals (Damietta, Idku), and domestic processing.

- Regulatory Framework: Petroleum sector reforms (2017–2020) improved IOC terms; recent challenges with gas pricing and payment arrears.

- Investment Attractiveness: Large market but macroeconomic challenges (currency crisis, debt) constrain new investment.

- Energy Security: Improved since Zohr but facing renewed import needs and foreign exchange constraints.

MOZAMBIQUE:

- Gas Reserves: ~2.8 Tcm (Rovuma Basin — among world's largest undeveloped)

- Gas-to-Power: Minimal domestic gas-to-power; focus on LNG exports (Coral FLNG, Mozambique LNG)

- Infrastructure: Limited domestic pipeline network; ROMPCO pipeline to South Africa (Pande-Temane fields declining from 2026).

- Regulatory Framework: Improving but challenged by insurgency in Cabo Delgado and governance concerns.

- Investment Attractiveness: Massive LNG projects attract investment but domestic utilization lags.

- Energy Security: Poor; despite massive reserves, most population lacks electricity access.

7. STRATEGIC RECOMMENDATIONS AND INTELLIGENCE OUTLOOK

7.1 Recommendations by Stakeholder

FOR THE GOVERNMENT OF GHANA:

- Prioritize ESRP implementation with measurable quartely milestones

- Establish an independent Gas Supply Security Task Force with authority to coordinate across GNPC, Ghana Gas, VRA, and ECG

- Fast-track the Petroleum Hub development at Atuabo to anchor industrial gas demand and FDI

- Implement the Energy Sector Levies (Amendment) Act 2025 revenue mechanisms transparently to build investor confidence

FOR THE MINISTRY OF ENERGY AND GREEN TRANSITION:

- Complete the National Gas Master Plan update with explicit supply security benchmarks and infrastructure sequencing

- Launch a competitive gas-to-industry auction mechanism to monetize stranded gas and attract petrochemical investment

- Accelerate renewable energy procurement (1.5 GW by 2030) to diversify generation and reduce gas dependence

- Develop a strategic gas storage policy, leveraging Tema LNG tankage as interim buffer

FOR GNPC:

- Expedite Pecan field development planning and Eban-Akoma appraisal

- Renegotiate gas supply agreements to introduce flexibility clauses for maintenance periods

- Enhance reservoir management on Sankofa to mitigate decline rates

- Develop domestic gas marketing capabilities to serve industrial offtakers

FOR GHANA GAS:

- Expand pipeline interconnectivity between western (Atuabo-Takoradi) and eastern (Tema) corridors

- Invest in compressor stations to improve pipeline throughput

- Develop gas quality monitoring and metering infrastructure

- Prepare for non-power gas demand growth (fertilizer, petrochemicals)

FOR VRA:

- Optimize hydro-thermal dispatch to minimize gas consumption during high-water periods

- Maintain dual-fuel readiness for supply disruption contingencies

- Collaborate with GRIDCo on transmission system upgrades

FOR PURC:

- Implement quarterly WACOG adjustments transparently to reflect cost changes

- Approve cost-reflective tariffs that balance affordability with sector financial viability

- Expedite smart metering rollout to reduce commercial losses

FOR PRIVATE INVESTORS:

- Evaluate opportunities in CCGT expansion (Sunon Asogli model), gas infrastructure, and industrial offtake

- Demand robust contractual safeguards (payment guarantees, escrow mechanisms) given sector debt risks

- Consider integrated gas-power-petrochemical investments at Atuabo Petroleum Hub

- Engage with development finance institutions for risk mitigation instruments

REFERENCES

African Development Bank. (2025). Ghana Country Focus Report 2025. Abidjan: AfDB.

Ecofin Agency. (2025, September 9). Ghana stabilizes power supply but faces $5.6 billion energy debt. Ecofin Agency.

Enerdata. (2026). Ghana Energy Information. Enerdata.

Energy Commission of Ghana. (2024, December 29). 2025 Energy Outlook for Ghana. Accra: Energy Commission.

Eni. (2025, March 19). Eni increases gas volumes for power generation in Côte d'Ivoire. Eni Press Release.

Ghana Grid Company. (2025). 2025 Electricity Supply Plan for Ghana. Accra: GRIDCo.

Global Legal Insights. (2025, December 22). Energy Laws and Regulations 2026: Ghana. Global Legal Insights.

GOGLA. (2024). Ghana Country Brief. Global Off-Grid Lighting Association.

IEA. (2025). Ghana — Natural Gas. International Energy Agency.

IEA. (2025). Energy Sub-Saharan Africa Programme in Ghana. International Energy Agency.

IRENA. (2025, March 26). Renewable Capacity Highlights 2025. Abu Dhabi: IRENA.

Ministry of Energy and Green Transition. (2025). 2025 Programme-Based Budget Estimates. Accra: MoEn.

Petroleum Commission of Ghana. (2025). Production Volume Data. Accra: PC.

PIAC. (2025, October 16). Ghana's crude oil output dips by 25.9% in first half of 2025. Public Interest and Accountability Committee.

PURC. (2025). Electricity Company of Ghana Ltd — Financial Analysis. Accra: PURC.

S&P Global Commodity Insights. (2025). Understanding Global LNG Markets and What It Means for Ghana's Energy Future. SPE Ghana.

Trade.gov. (2025, March 20). Ghana Power Generation Outlook for 2025. U.S. Department of Commerce.

World Bank. (2024). Ghana Energy Sector Recovery Program (P173258) — Program Appraisal Document. Washington, DC: World Bank.

World Bank. (2024). Ghana — Share of Industry in GDP. World Bank Open Data.