# Senegal LNG Export & Gas Monetization

# Executive Summary

Senegal has emerged as one of Africa's most promising new hydrocarbon producers, transitioning from a net energy importer to an LNG exporter in under two years. The Greater Tortue Ahmeyim (GTA) Phase 1 project, a $4.8 billion cross-border development between Senegal and Mauritania, achieved first gas in December 2024, first LNG in February 2025, and is now ramping toward full commercial operations. With 2026 exports projected to nearly double to 36 LNG cargoes (~2.7 mtpa), Senegal is positioning itself as West Africa's newest energy hub. Simultaneously, the government is pursuing an ambitious domestic gas monetization agenda centered on the Yakaar-Teranga field (25 Tcf), which has shifted from an export-focused LNG project to a domestic-energy-first strategy under state-owned Petrosen.

# Country Natural Gas Reserve

Senegal's offshore gas endowment is substantial and growing, with multiple discoveries across the MSGBC Basin (Mauritania-Senegal-Guinea Bissau-Conakry). The Tortue field alone contains an estimated 15 Tcf of recoverable gas resource potential, with the broader basin thought to contain up to 100 Tcf of gas.

The Yakaar-Teranga discovery ( extending ~200 km from Mauritania through the GTA area into Senegal), confirmed by Kosmos Energy and BP in 2019, represents Senegal's largest single gas resource at approximately 25 Tcf.

| Field | Estimated Reserves | Status | Primary Use |
| --- | --- | --- | --- |
| Greater Tortue Ahmeyim | 15 Tcf recoverable (50% Senegal share) | Producing (Phase 1) | LNG Export + Domestic |
| Yakaar-Teranga | 25 Tcf recoverable | Under State Development | Domestic Priority |
| Sangomar (Gas) | Approximately 4 Tcf | Producing (Oil-first) | Associated Gas |
| Other Prospects | Approximately 10 Tcf (estimated) | Exploration | TBD |

Senegal’s Gas Business Operators

| Company/Entity | Country | Involvement | Contribution |
| --- | --- | --- | --- |
| BP | UK | GTA Operator (56%), LNG Buyer | $4.8B project leadership, FLNG technology |
| Kosmos Energy | USA | GTA (27%), Former Yakaar Operator | Deepwater exploration expertise |
| Woodside Energy | Australia | Sangomar Operator (68%) | FPSO technology, offshore development |
| MCC (USA) | USA | $550M+ energy compact | Grid modernization, institutional support |
| FONSIS | Senegal | Sovereign Wealth Fund | Domestic infrastructure financing |

# Senegal's Demand and Supply Dynamics

- Electricity Demand Surge

Senegal's electricity demand is accelerating rapidly. Consumption rose 22% year-on-year in 2025, and the IEA projects approximately 8% annual growth through 2030. Senegal currently has some of Africa's highest generation costs (34–38 ¢/kWh) due to dependence on imported liquid fuels for approximately 90% of power generation.

| Metric | 2024 | 2025 | 2030 (Proj) |
| --- | --- | --- | --- |
| Electricity Access | 84% | ~86% | Universal (100%) |
| Installed Capacity | ~1.8 GW | ~1.9 GW | ~3.0 GW |
| Gas Share in Generation | ~25% | ~26% | ~30% |
| Renewables Share | ~20% | ~21% | ~22% |
| Heavy Fuel Oil Share | ~44% | ~43% | ~20% |

# Infrastructure and Pipeline Network

Senegal is building a 400-kilometer national gas pipeline network (Réseau Gazier du Sénégal) to connect offshore fields to domestic demand centers.

RGS Pipeline Segments

| Segment | Length | Capacity | Est. Cost | Route |
| --- | --- | --- | --- | --- |
| Northern | 85 km | 300 MMscf/d | €275M ($298M) | GTA Hub → Gandon Power Plant |
| Green | 110 km | 300 MMscf/d | €183M ($198M) | Northern-Southern Interconnect |
| Blue | 100 km | 713 MMscf/d | €214M ($232M) | Mboro → Cap des Biches |
| Orange | 45 km | 300 MMscf/d | €153M ($166M) | Sendou → Malicounda |
| Red | 17 km | 150 MMscf/d | €150M ($163M) | Branch Line |

Key Infrastructure Milestones

- May 2026: CRD approves GTA-Gandon pipeline construction start

- 2027: National gas pipeline inauguration target

- Connection to WAGP: Designed to link with the 5,700 km African Atlantic Gas Pipeline

# Senegal's Gas Monetization Technologies

- Current Technologies Deployed

| Technology | Application | Project | Status |
| --- | --- | --- | --- |
| FLNG (Floating LNG) | Offshore liquefaction | GTA Phase 1 (Gimi vessel) | Operating |
| FPSO | Gas pre-treatment | GTA (80 km subsea tie-back) | Operating |
| Subsea Production System | Deepwater extraction | GTA (2,800m depth) | Operating |
| Hub Terminal/Breakwater | LNG carrier shelter | GTA nearshore | Operating |

- Emerging/Planned Technologies

| Technology | Planned Application | Target |
| --- | --- | --- |
| Gravity-Based LNG Structure | Phase 2 expansion | +2.5–3 mtpa capacity |
| Onshore Pipeline Network | Domestic gas distribution | 2.5 bcm/year by 2027 |
| CCGT Power Plants | Gas-to-power conversion | 250–500 MW new capacity |
| FLNG (Yakaar) | Cancelled — shifted to domestic pipeline | N/A |

# Investment Opportunities

- Deepwater exploration within the MSGBC Basin, where significant untapped gas potential remains.

- Brownfield expansion around Sangomar and future development phases.

- LNG value chain expansion through future GTA phases and associated infrastructure.

- Potential farm-in opportunities as Petrosen seeks technical and financial partners for Yakaar-Teranga.

# Conclusion

Senegal stands at an inflection point. The GTA project has successfully delivered Africa's newest LNG export stream, with 2026 poised to be a record year. However, the more transformative story may be unfolding onshore: the Yakaar-Teranga domestic pivot and the national gas pipeline buildout represent a fundamental reorientation from "resource extraction for export" to "resource utilization for development." For investors, this creates a dual-track opportunity — upstream LNG cash flows near-term, and downstream gas-to-power/industrial growth medium-term. The critical variable is execution: whether Petrosen and its partners can deliver the $7.5B Yakaar project and $1.15B pipeline network on time and on budget. Success would cement Senegal as West Africa's model for gas-as-transition — monetizing resources for both global markets and domestic prosperity.